304-DPW-016 HMA PATCHING

The Standard Specifications are revised as follows:

SECTION 304, AFTER LINE 16, INSERT AS FOLLOWS:

Areas to be patched shall be saw cut full depth prior to removal of the existing material.  Failed material shall be removed to the minimum depth as shown on the detail sheets.  If suitable subgrade cannot be obtained at the planned depth for repair, the Engineer shall be notified prior to any additional excavation.  Payment for material placed in excess of planned quantity will not be made without prior written approval from Owner.

Patching shall be completed after milling operations unless otherwise noted. If patching is to be completed prior to milling, as approved by the Engineer, a transition shall be required and the depth of the transition shall not exceed 1 in. (max.) below existing or finished surface and any additional base material placed to achieve the transition will be paid for as HMA Patching. No additional payment for Mobilization and/or Demobilization shall be made for additional HMA Patching required after milling.

______________________________________________________________________________